Setingan Dasar MikroTik UKK 2024

IP  >>  DHCP Client >> (+) >> Interface = ether 1

IP >> DNS >> () >> Allow remote request

IP >> Firewal >> NAT  >> (+) >> Chain = srcnat, out interface = ether 1

New terminal >>  ping 8.8.8.8

System >> SNTP Client  >> () Enable ,  Primary NTP Server : 0.id.pool.ntp.org,  Secondary NTP Server :  1.id.pool.ntp.org

System >> Clock >> Time : (di cocokkan) >> Date : (di cocokkan) >> Time Zone Name : Asia/jakarta

Seting LAN 2

IP >> Address >> (+) >> Address = 192.168.100.1/25, interface : ether 2

IP >> DHCP Server >> Server >> DHCP Setup >> DHCP Server interface : ether 2 >> address to give out = 192.168.100.2-192.168.100.100  >> next sampai selesai.

IP >> Firewall >> Filter Rules >> (+) >> chain = input, src Addres = 192.168.100.2-192.168.100.50, dst addres 192.168.100.1/25, protocol = icmp >> action = drop

IP >> Firewall >> Filter Rules >> (+) >> Chain = forward, src Address = 192.168.100.51-192.168.100.100, dst addres 192.168.200.0/24, protocol = icmp >> action = drop

IP >> Firewal >> Filter Rules >> (+) >> Chain = input  >> Action >> action = log >> () log >> log prefix : (akses-ke-router)

Sistem >> logging >> (+) >> prefix : (akses-ke-router), action : disk

Seting Wifi

Wiriless >> Wifi Interfaces  >> () Wlan 1 >> (klik 2x) Wlan 1 >>  Wireless >> mode = ap bridge, SSID = Nama_perserta@proxyUKK

IP >> Address >> (+) >> Address = 192.168.200.1/24, interface : wlan1

IP >> Hotspot >> Server >> Hotspot Setup >>  hotspot interface : wlan1 >> address pool of network = 192.168.200.2-192.168.200.100  >>  DNS name = smknh.net

Buat Akun User Hostpot Sebanyak 20

Version 6.47.9 Mipsbe

Download file all page di : https://mikrotik.com/download/archive (sesuaikan dengan versi mikrotik).

IP >> Address >> (+) >> Address = 192.168.60.1/24, interface : ether 5

IP >> DHCP Server >> Server >> DHCP setup >> DHCP server interface : ether 5 >> next sampai selesai

IP >> Hospot >> Server Profile >> Klik 2x hsprof1 >> Radius  >> () use Radius

Radius >> incoming >> () accept

Radius >> (+) >> () hostpot , address : 192.168.60.1, secret : 123

Buka google chrome >> http://192.168.60.1/userman

Isi login admin lalu enter

Routers >> Add >> New >> name : (Nama Siswa), IP Address : 192.168.60.1, secret : 123 >> add

Profiles >> limitations >> add >> new >> name : nama siswa >> add

Profile >> Profile >> (+) >> name : profile1 >> create >> add new limitation >> time : 07:00:00 - 16:00:00, () nama siswa >> add

Users >> add >> batch >> nember of users : 20, username langth : 3, pasword length : 3 >> add

Blokir Situs Dengan Web Proxy

IP >> Firewall >> Chain : dstnat, Protocol : 6 (tcp), dst-port : 80 >> action  >> action : redirect >> to ports : 8080

IP >> Web Proxy >> () enable, port : 8080, cache administrator : nama_peserta@sekolah.sch.id, () cache on disk >> apply >> access >> (+) >> dst-port : 80, dst-host : *.linux.*, action : deny

IP >> Web Proxy >> access >> (+) >> dst-port : 80, patch : *.mp3*, action : deny

IP >> Web Proxy >> access >> (+) >> dst-port : 80, patch : *.mkv*, action : deny